DESIGN AND LTspice SIMULATION OF A
VOLTAGE-DIVIDER BIASED COMMON-EMITTER BJT AMPLIFIER

Project Design, Biasing, Transient Analysis, Troubleshooting and Verification

Topic: Common-emitter BJT voltage amplifier using a 2N3904 and LTspice

Purpose: Design, simulate, verify and troubleshoot a small-signal common-emitter amplifier

Simulation platform: LTspice

Report date: 25 August 2026

Name : Yogeswaran K

Reg No : 192412615

# 1. Executive Summary / Overview

This project studies a single-stage common-emitter (CE) BJT amplifier using a 2N3904 transistor in LTspice. The original circuit supplied for review contains a 12 V source, 4 kΩ collector resistor, 1 kΩ emitter resistor, 10 µF output coupling capacitor, 2 kΩ load and an input source defined as SINE(0 10m 1K).

The central finding is that the original circuit has no independent DC bias path for the transistor base. Because SINE(0 10m 1K) has zero DC offset, the base is approximately 0 V DC and the 2N3904 is driven toward cutoff. This explains the microvolt-level output seen in the supplied LTspice waveforms.

The recommended correction is a voltage-divider bias network using 82 kΩ from +12 V to the base and 15 kΩ from the base to ground, plus a 10 µF input coupling capacitor. First-order calculations give VB≈1.86 V, VE≈1.16 V, IE≈1.16 mA, VC≈7.36 V and VCE≈6.20 V. These are design estimates; the final values must be verified in LTspice.

# 2. Project Definition

## 2.1 Exact Topic

Design and LTspice simulation of a voltage-divider biased single-stage common-emitter BJT amplifier using a 2N3904 transistor.

## 2.2 Purpose

Understand the DC bias requirements of a common-emitter BJT amplifier.

Build and verify the circuit in LTspice.

Apply a 10 mV, 1 kHz small-signal input and observe the output.

Diagnose the original microvolt-level output.

Introduce proper voltage-divider biasing and coupling capacitors.

Compare hand calculations with LTspice operating-point results.

Provide a beginner-friendly verification and troubleshooting procedure.

## 2.3 Target Audience

Beginner electronics students, project demonstrators, laboratory instructors and evaluators.

## 2.4 Content Requirements Included

Original circuit and component values; theory; biasing problem; corrected design; calculations; LTspice procedure; waveform interpretation; troubleshooting; verification checklist; glossary; references/notes; appendices.

# 3. Original LTspice Circuit

## 3.1 Component Inventory

| Reference | Component | Value / setting | Function |
| --- | --- | --- | --- |
| V1 | DC source | 12 V | Collector supply |
| V2 | Sine source | SINE(0 10m 1K) | AC input |
| Q1 | NPN BJT | 2N3904 | Amplifying device |
| RC | Resistor | 4 kΩ | Collector resistor |
| RE (shown as R2 originally) | Resistor | 1 kΩ | Emitter resistor |
| C2 | Capacitor | 10 µF | Output coupling |
| RL | Resistor | 2 kΩ | Load |
| Transient command | .tran | 10m | 10 ms simulation |

## 3.2 Original Schematic

Figure 1. Original LTspice schematic supplied for the project.

The collector is connected to +12 V through RC, the emitter is connected to ground through the 1 kΩ resistor, and the collector is coupled through C2 to the 2 kΩ load. The input source is directly connected to the base.

## 3.3 What Is Correct

12 V supply is referenced to ground.

RC=4 kΩ is placed between supply and collector.

RE=1 kΩ is returned from emitter to ground.

C2=10 µF couples the collector to the load.

RL=2 kΩ is returned to ground.

Q1 is a 2N3904 NPN transistor.

## 3.4 Critical Problem

The base has no independent DC bias network. The source SINE(0 10m 1K) is centered at 0 V, so the base does not receive the DC voltage needed to establish a useful forward-active Q-point.

The transistor is near cutoff.

A 10 mV AC signal alone does not establish the required DC operating point.

The collector can remain close to the supply voltage while collector current is very small.

The output can therefore be only a few microvolts.

# 4. Simulation Observations from Supplied Runs

The observations below are based on the LTspice screenshots supplied with the project. LTspice node names such as V(n002), V(n003) and V(n005) are automatically generated and should be confirmed by clicking the actual wire.

| Input / condition | Visible result | Interpretation |
| --- | --- | --- |
| SINE(0 10m 1K) | Output only a few µV | No DC base bias; transistor near cutoff. |
| V(n005) with zero-DC input | Approximately ±5 µV waveform | Extremely small response around an unsuitable Q-point. |
| SINE(0.1 1K) | Larger visible waveforms | Adding DC offset begins to turn the transistor on. |
| SINE(1 1K) | Hundreds of mV and nonlinear-looking waveforms | DC level and AC amplitude are too large for clean small-signal operation. |
| V(n002) readout | DC operating point about 524 mV | A biased node is visible in that run; exact terminal identity should be verified in the schematic. |

Conclusion from the supplied evidence: changing the source DC offset changes transistor conduction, confirming that the missing DC bias is the main issue.

# 5. Common-Emitter Amplifier Theory

## 5.1 Operating Principle

In a common-emitter amplifier, a small change in base-emitter voltage causes a change in collector current. The collector resistor converts that current change into an output-voltage change. The stage normally provides voltage gain and inverts the signal.

## 5.2 Operating Regions

Cutoff: transistor current is very small; the device is effectively OFF.

Forward-active: suitable region for approximately linear small-signal amplification.

Saturation: transistor is strongly ON; collector-emitter voltage is small and distortion can occur.

## 5.3 Why Biasing Matters

The transistor needs a quiescent operating point before the AC signal is applied. A suitable Q-point provides room for the collector voltage to move up and down without immediately reaching cutoff or saturation.

## 5.4 Roles of RE, C1 and C2

RE provides emitter degeneration/negative feedback and improves DC stability.

C1 isolates the source DC level from the transistor's bias voltage.

C2 blocks the collector's DC voltage from the load while passing the AC output.

# 6. Corrected Circuit Design

## 6.1 Recommended Values

| Component | Value | Connection |
| --- | --- | --- |
| VCC | 12 V | +12 V supply rail |
| RC | 4 kΩ | +12 V to collector |
| RE | 1 kΩ | Emitter to ground |
| R1 | 82 kΩ | +12 V to base |
| R2 | 15 kΩ | Base to ground |
| C1 | 10 µF | Input source to base |
| C2 | 10 µF | Collector to output |
| RL | 2 kΩ | Output to ground |
| V2 | SINE(0 10m 1K) | 10 mV peak, 1 kHz input |

## 6.2 Connection Sequence

Keep V1 at 12 V DC and connect its negative terminal to ground.

Connect RC=4 kΩ from +12 V to the collector of Q1.

Keep Q1 as 2N3904.

Connect RE=1 kΩ from emitter to ground.

Add R1=82 kΩ from +12 V to the base.

Add R2=15 kΩ from base to ground.

Insert C1=10 µF between V2 and the base.

Set V2 to SINE(0 10m 1K).

Keep C2=10 µF from collector to the output node.

Connect RL=2 kΩ from output to ground.

Use .tran 10m for the first transient run.

## 6.3 Connection Map

Supply: +12 V → RC → collector → Q1 → emitter → RE → ground.

Base bias: +12 V → R1 → base → R2 → ground.

Input: V2 → C1 → base.

Output: collector → C2 → output node → RL → ground.

# 7. DC Bias Calculations

## 7.1 Base Voltage

Using the unloaded voltage-divider approximation:

VB ≈ VCC × R2/(R1+R2)

VB ≈ 12 × 15k/(82k+15k) ≈ 1.86 V

## 7.2 Emitter Voltage

VE ≈ VB − VBE ≈ 1.86 − 0.70 ≈ 1.16 V

## 7.3 Emitter and Collector Current

IE ≈ VE/RE ≈ 1.16 V/1 kΩ ≈ 1.16 mA

IC ≈ IE ≈ 1.16 mA  (first-order approximation)

## 7.4 Collector Voltage

VC ≈ VCC − IC×RC ≈ 12 − (1.16 mA)(4 kΩ) ≈ 7.36 V

## 7.5 Collector-Emitter Voltage

VCE ≈ VC − VE ≈ 7.36 − 1.16 ≈ 6.20 V

| Quantity | Approx. value | Meaning |
| --- | --- | --- |
| VCC | 12 V | Supply |
| VB | 1.86 V | Base bias |
| VE | 1.16 V | Emitter DC voltage |
| IE | 1.16 mA | Emitter current |
| IC | ≈1.16 mA | Collector current estimate |
| VC | ≈7.36 V | Collector DC voltage |
| VCE | ≈6.20 V | Q-point voltage headroom |

These calculations are first-order estimates. LTspice includes the actual transistor model and base-current loading, so the simulated values will not be exactly identical.

# 8. LTspice Simulation Procedure

## 8.1 Build and Label

Place V1, V2, Q1, RC, RE, R1, R2, C1, C2 and RL.

Use ground symbols for every required return path.

Check that every wire endpoint actually touches a component pin.

Use net labels BASE, COLLECTOR and OUTPUT if desired.

Ensure the emitter resistor is uniquely named RE and the bias resistors are R1 and R2.

## 8.2 Source Settings

V1 = 12 V DC.

V2 = SINE(0 10m 1K).

0 = DC offset, 10m = 10 mV peak amplitude, 1K = 1 kHz.

Do not use a 1 V DC offset as the normal small-signal input; establish bias with the resistor network.

## 8.3 Transient Analysis

.tran 10m

At 1 kHz, 10 ms contains about ten cycles, enough for an initial waveform check.

## 8.4 Waveforms to Plot

Input/source node.

Base node to confirm DC bias.

Collector node to observe amplified and inverted collector waveform.

Output node across RL after C2.

Optional emitter node for checking emitter degeneration.

# 9. Troubleshooting and Verification

## 9.1 Microvolt Output Diagnosis

The microvolt output in the original simulation is consistent with a transistor that is not correctly DC biased. The correct response is not to arbitrarily increase the source offset; instead, establish the Q-point with a dedicated bias network.

## 9.2 Why SINE(1 1K) Is Not the Correct Fix

A 1 V DC offset changes the transistor operating point through the signal source itself, and a 1 V AC amplitude is very large for a small-signal amplifier. This can create nonlinear conduction and clipping. It is useful as a diagnostic experiment, but it should not be treated as the final biasing method.

## 9.3 Common LTspice Checks

Component pin not actually connected.

Missing ground reference.

Wrong transistor model or symbol.

Incorrect source DC offset.

Incorrect transient command.

Plotting the wrong node.

Output measured before the intended coupling capacitor.

Confusing resistor reference designators.

## 9.4 Verification Checklist

| Check | Expected | Record |
| --- | --- | --- |
| VCC | 12 V | Pass / Fail |
| VB | ≈1.86 V | Pass / Fail |
| VE | ≈1.16 V | Pass / Fail |
| VC | ≈7.36 V | Pass / Fail |
| VCE | ≈6.20 V | Pass / Fail |
| Input | 10 mV peak, 1 kHz | Pass / Fail |
| Output | Clear AC waveform | Pass / Fail |
| Phase | Approximately inverted | Pass / Fail |
| Distortion | Low for small input | Pass / Fail |

# 10. Expected Results and Discussion

After correction, the transistor should have a stable DC Q-point. The 10 mV, 1 kHz input will then produce a small AC variation around that point. The collector/output signal should be larger than the input for a functioning voltage amplifier and should be approximately 180° out of phase.

Do not claim a final voltage gain from the original screenshots. Measure the corrected circuit directly:

Av = Vout(peak-to-peak) / Vin(peak-to-peak)

The exact gain depends on the transistor model, bias point, emitter degeneration, source impedance, load and capacitor reactances. The final project result should therefore use the measured LTspice waveform values.

# 11. Recommended Project Workflow

Rebuild the corrected circuit exactly.

Run DC operating-point analysis and record VB, VE, VC and VCE.

Compare simulated values with Section 7.

Run .tran 10m with SINE(0 10m 1K).

Plot input, base, collector and output.

Measure input and output peak-to-peak amplitudes.

Calculate voltage gain.

Check phase inversion.

Check for clipping or distortion.

Capture final schematic and waveform screenshots.

# 12. Conclusion

The original LTspice circuit has the correct broad common-emitter topology but lacks the DC base-bias network required for a reliable amplifier operating point. The supplied microvolt-level output is therefore a useful diagnostic result rather than an LTspice failure.

The recommended final circuit adds an 82 kΩ/15 kΩ voltage-divider bias network and a 10 µF input coupling capacitor while retaining the 12 V supply, 4 kΩ collector resistor, 1 kΩ emitter resistor, 10 µF output coupling capacitor and 2 kΩ load. First-order calculations predict a base voltage of about 1.86 V and a collector-emitter voltage of about 6.20 V. The final design must be verified with LTspice operating-point and transient analyses.

# 13. Glossary

| Term | Meaning |
| --- | --- |
| AC | Time-varying signal component. |
| Biasing | Applying DC voltages/currents to establish a transistor operating point. |
| BJT | Bipolar Junction Transistor. |
| Common-emitter | BJT configuration using the emitter as the common reference. |
| Cutoff | Region in which transistor current is very small. |
| Forward-active | BJT region used for approximately linear amplification. |
| LTspice | Circuit simulation software used for this project. |
| Q-point | Quiescent DC operating point before the AC signal. |
| Saturation | Region where the transistor is strongly ON and linear amplification is lost. |
| Transient analysis | Simulation of circuit behavior versus time. |
| Voltage-divider bias | Two-resistor network used to establish base DC voltage. |

# 14. References and Technical Notes

Reference Note 1. Original circuit values and source expression are transcribed from the LTspice schematic supplied in this conversation: V1=12 V, RC=4 kΩ, emitter resistor=1 kΩ, C2=10 µF, RL=2 kΩ, Q1=2N3904 and V2=SINE(0 10m 1K).

Reference Note 2. Simulation observations are based on the supplied LTspice screenshots, including the visible waveform scales and operating-point readouts.

Reference Note 3. Bias calculations are first-order estimates using a nominal VBE of about 0.70 V. Actual LTspice results depend on the 2N3904 model and base-current loading.

Reference Note 4. The final report should quote measured values from the corrected LTspice simulation for voltage gain, phase, distortion and exact Q-point.